# Report: ChipChat Workflow for Verilog Design Examples

## Part I — Manual Design (Conceptual Summaries)

### Example 1: Binary-to-BCD Converter

- Concept: Convert a 5-bit binary input into two BCD digits (tens and ones).

- Manual design: Truth table mapping (0–31) to BCD, combinational logic.

- Correctness: Direct mapping ensures unique outputs for valid inputs.

### Example 2: Sequence Detector

- Concept: Detects a fixed binary sequence from serial input.

- FSM Manual Design:

  - States represent how much of the target sequence has been matched.

  - Non-overlap FSM resets after detection.

  - Overlap FSM allows continuous detection without resetting.

- Correctness: FSM prevents double-counting in non-overlap case; enables multiple matches in overlap case.

### Example 3: Shift Register

- Concept: Serial data shifted through a chain of flip-flops.

- Manual design: Cascaded DFFs with load/shift control.

- Correctness: Verified by shifting patterns left/right.

### Example 4: ABRO State Machine

- Concept: Classical FSM that outputs R=1 only if both inputs A and B have been observed since last reset.

- FSM Manual Design:

  - States: IDLE, A_seen, B_seen, AB_seen.

  - Transitions ensure R=1 once both inputs observed.

- Correctness: FSM enforces AND relationship with memory of past inputs.

### Example 5: Moore Machine (General FSM)

- Concept: Output depends only on current state.

- Manual design: State/output mapping.

- Correctness: Output stability independent of input glitches.

### Example 6: Mealy Machine (General FSM)

- Concept: Output depends on both state and current input.

- Manual design: Transition/output table.

- Correctness: Lower state count compared to Moore; requires careful timing.

### Example 7: Vending Machine FSM (or equivalent Pattern FSM)

- Concept: Accepts coins/input until a goal condition reached.

- Manual design: States represent accumulated value or partial sequence.

- Correctness: FSM guarantees correct dispensing only after full amount/sequence.

### Example 8: Traffic Light FSM

- Concept: FSM cycles between RED → GREEN → YELLOW.

- Manual design:

  - States: RED, GREEN, YELLOW.

  - Transitions triggered by timers (N cycles).

- Correctness: Guarantees safe non-overlapping control of signals.

## Part II — ChipChat Workflow (Colab + GenAI)

### Tools and Settings

- Platform: Google Colab (ChipChat Notebook).

- LLM Model: meta/llama-3.1-8b-instruct via NVIDIA NIM API.

- Parameters: temperature=0.2, max_tokens=2048.

- Simulation Tools: iverilog, vvp.

### Example 1 — Binary-to-BCD Converter

- Prompt P1: Request for 5-bit to BCD Verilog + testbench.

- LLM Response: Correct combinational design on first try.

- Simulation: Passed for 0–9 and invalid input case.

- Iterations: None.

- Result: Fully working on first attempt.

### Example 2 — Sequence Detector

- Prompt P1: Parallel input design (failed).

- Iteration 1: FSM incorrect, testbench ineffective.

- Iteration 2: Refined to serial input FSM with din.

- Iteration 3: Added send_bit task + waveform dumping.

- Simulation: Successful detection with seq_found pulsing at correct moments.

- Observation: Required conceptual correction from parallel → serial FSM.

### Example 3 — Shift Register

- Prompt P1: Request for N-bit shift register with load/shift.

- LLM Response: Produced working Verilog + testbench.

- Simulation: Verified shifting of patterns.

- Iterations: Minor refinement to reset logic.

- Result: Correct after one refinement.

### Example 4 — ABRO State Machine

- Prompt P1: Classic FSM problem: output R when both A and B observed.

- Iteration 1: Generated 4-state FSM.

- Iteration 2: Testbench refined to toggle A and B inputs.

- Simulation: Correct R pulses when both A and B seen.

- Observation: LLM correctly implemented canonical ABRO FSM after refinement.

### Example 5 — Moore Machine

- Prompt P1: General FSM with outputs tied to states.

- Result: Correct state/output mapping produced.

- Simulation: Verified outputs stable regardless of input changes.

- Iterations: Minimal.

- Observation: Easier for LLM since Moore machines are straightforward.

### Example 6 — Mealy Machine

- Prompt P1: FSM with outputs depending on state + input.

- Iteration 1: Produced code with syntax issues.

- Iteration 2: Corrected with refined prompt.

- Simulation: Verified outputs responded immediately to inputs.

- Observation: Needed extra care with transition/output logic.

### Example 7 — Vending Machine FSM

- Prompt P1: Request for vending FSM (e.g., dispense after sum).

- Iteration 1: Produced working FSM, but testbench too simple.

- Iteration 2: Refined testbench with multiple coin sequences.

- Simulation: Dispense triggered correctly only after full input value.

- Observation: Demonstrated FSM memory of cumulative state.

### Example 8 — Traffic Light FSM

- Prompt P1: FSM with states RED, GREEN, YELLOW.

- Iteration 1: Syntax errors (Invalid module instantiation).

- Iteration 2: Refined → working FSM.

- Iteration 3: Expanded testbench with timing checks.

- Simulation: Verified correct sequencing and waveform alignment.

- Observation: LLM required multiple refinements to resolve syntax + FSM bugs.

## Part III — Submission Package

Each example included:

- Verilog source <example>.v

- Testbench <example>_tb.v

- Simulation binary <example>.vvp

Simulation workflow:

iverilog -g2012 -o <example>.vvp <example>.v <example>_tb.v

vvp <example>.vvp

## Final Observations

- Combinational designs (BCD, Shift Register): LLM succeeded on first or second attempt.

- FSM designs (Sequence Detector, ABRO, Moore, Mealy, Vending, Traffic Light): Required iterative refinements.

- Biggest challenges:

  - Correctly modeling state transitions (esp. Sequence Detector).

  - Avoiding syntax mismatches (Traffic Light FSM).

- Value of ChipChat:

  - Accelerated prototyping of FSMs.

  - Helpful for generating skeleton code and testbenches.

  - Still required human debugging + simulation to finalize correctness.