# Report: AutoChip Tutorial

## Part I — Introduction

AutoChip is a GenAI-assisted framework for automatic Verilog design generation and correction. Unlike conversational tools such as ChipChat, AutoChip follows a structured error-aware loop:

- Generate Verilog from a design prompt.

- Compile with iverilog.

- If errors occur, feed compiler/testbench messages back to the LLM.

- Iterate until the design compiles and passes simulation.

This makes AutoChip particularly suited for FSMs and sequential circuits, where compilation correctness is essential. However, its effectiveness depends on clear constraints and guardrails.

## Part II — Workflow

### Tools & Environment

- Platform: Google Colab (Jupyter Notebook).

- LLM API: NVIDIA NIM endpoint via OpenAI Python client.

- Model: meta/llama-3.1-8b-instruct

- Simulation: iverilog (compile), vvp (simulate), gtkwave (waveforms).

- Parameters:

  - temperature = 0.2

  - max_tokens = 1200

### Cost Control Modification

To avoid higher API charges, I intentionally modified the model type to a smaller, cheaper model (meta/llama-3.1-8b-instruct) and disabled ensemble calls. This kept iteration costs minimal and eliminated the “ensemble not deployed” error.

## Part III — Examples

### Example 1 — Simple Combinational Logic (7420 Chip)

- Spec: Implement 7420-like dual 4-input NAND.

- Iteration 1: Produced correct combinational code.

- Simulation: iverilog -g2012 -o 7420.vvp 7420.v 7420_tb.v && vvp 7420.vvp → passed testbench.

- Observation: For purely combinational logic, AutoChip succeeded immediately.

### Example 2 — Arbiter FSM

- Spec: Arbiter circuit (grant among multiple requesters).

- AutoChip Results:

Iterations 0–7: Compilation errors

Iteration 8: Simulation error — 939 mismatches (1054 samples)

Iteration 9: Compilation error

Iteration 10: Simulation error — 863 mismatches (1054 samples)

- Analysis:

  - Early failures: LLM repeatedly generated SystemVerilog constructs (e.g., logic, enum) or multi-module code incompatible with Icarus.

  - Later runs compiled but did not match the test oracle. Likely cause: arbiter policy mismatch (round-robin vs static priority, or reset behavior).

- Outcome: Never passed simulation.

- Next Steps:

  - Enforce stricter prompt guardrails: “Verilog-2001 only, no SV features, one module only.”

### Example 3 — Sequential Logic (Rule-90 Cellular Automaton)

- Spec: Implement 1-D Rule-90 CA, each cell’s next state = XOR(left, right).

- AutoChip Results:

Iteration 0 → Compilation error

Iteration 1 → Compilation error

Iteration 2 → Compilation error

Iteration 3 → Compilation error

Iteration 4 → Compilation error

Iteration 5 → Compilation error

- Analysis:

  - AutoChip never progressed past compilation.

  - Unlike Example 2, no simulation was reached.

- Outcome: Failed all iterations.

- Next Steps:

  - Provide a strict Verilog-2001 skeleton (with parameterized width, XOR logic, synchronous reset).

  - Add a syntax filter rejecting logic, enum, etc., before sending code to the compiler.

## Part IV — Error Analysis

- Example 2 (Arbiter): Demonstrated AutoChip’s compilation + functional fragility. It compiled only intermittently, and simulation mismatched due to policy divergence.

- Example 3 (Rule-90): Demonstrated AutoChip’s syntax fragility. Without constraints, the loop retried the same class of compilation errors indefinitely.

## Part V — Observations

### Strengths

- Structured retry loop automates correction of some compilation errors.

- Logging & reproducibility: each iteration saved with cost/response info.

- Good for combinational circuits (Example 1 succeeded immediately).

### Weaknesses

- FSM and sequential logic: prone to repeated compilation errors (Examples 2, 3).

- Policy mismatch: simulation errors persist if LLM chooses a different FSM semantics than oracle.

- No syntax awareness: repeats invalid SV constructs without guardrails.

### Cost Optimization

- Switched from ensemble route to a single smaller model.

- Disabled ensemble mode, preventing the “ensemble is not deployed” error.

- Verified each model ID with a simple completion before running the AutoChip loop.

## Part VI — Submission Package

For each example directory (7420_example/, arbiter_example/, rule90_example/):

- AutoChip_Tutorial_ETS2025.ipynb — edited notebook

- config.json — configurations

Report: AutoChip_Report.pdf (this document).